Team 2:

DataCom: OSPF, EIGRP and GRE Tunnels

NMS: NMP-based Network Management System using OpenNMS, RESTCONF, NETCONF

TEAM 2 PROJECT

This project demonstrates the implementation of enterprise routing protocols and secure tunneling in a simulated network environment, along with NETCONF-based automation.

Key Technologies Implemented:

- Multi-Area OSPFv2 (IPv4)
 Link-state routing with backbone and non-backbone areas

- OSPFv3 (IPv6)
 IPv6 routing and neighbor establishment

- EIGRP AS 100 Configuration
 Hybrid routing protocol with fast convergence

- GRE Tunnel over OSPF
 Virtual point-to-point tunnel for secure reachability

- NETCONF Automation using Python
 Automated OSPFv2 configuration push using ncclient

Simulation Tool

- GNS3 for network design, configuration, and testing

TOPOLOGY :

Network Addressing Plan and Interface Allocation

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R3 | Gi3 (LAN) | 192.168.1.1 | 255.255.255.0 |
| R3 | Gi2 (to R1) | 192.168.2.1 | 255.255.255.252 |
| R1 | e1/0 (to R3) | 192.168.2.2 | 255.255.255.252 |
| R1 | e1/1 (to R2) | 192.168.3.1 | 255.255.255.252 |
| R2 | e1/0 (to R1) | 192.168.3.2 | 255.255.255.252 |
| R2 | e1/1 (LAN) | 192.168.4.1 | 255.255.255.0 |

## IPv4 Addressing Plan for PCs:

- Left LAN (PC1–PC2–PC5 Network): 192.168.1.0/24

- Right LAN (PC3–PC4–PC6 Network): 192.168.4.0/24

Router Interface IP Configuration

MULTI-AREA OSPFv2 (IPv4)

### Multi-Area OSPF is an OSPF design where the network is divided into multiple smaller areas instead of using a single large area.

It improves scalability and performance by reducing routing overhead

## Key Points

- Area 0 is the Backbone Area

- Other areas (Area 1, Area 2, etc.) connect to Area 0 through ABRs (Area Border Routers)

- LSAs are limited within an area, reducing LSDB size

- Faster convergence and efficient routing in large enterprise networks

Area Assignment

- Left LAN → Area 1

- Backbone → Area 0

- Right LAN → Area 2

R3 (Area 1)

R1 (Area 0)

R2 (Area 2)

Verify OSPF

FULL OSPFv3 (IPv6)

| Link | IPv6 Network |
| --- | --- |
| Left LAN | 2001:DB8:1::/64 |
| R3 ↔ R1 | 2001:DB8:12::/64 |
| R1 ↔ R2 | 2001:DB8:23::/64 |
| Right LAN | 2001:DB8:4::/64 |
| Configure OSPFv3 between the 3 routers |  |

Verification

EIGRP AS 100 (3 Routers + PCs)

EIGRP (Enhanced Interior Gateway Routing Protocol) is a Cisco-developed dynamic routing protocol used to share routing information between routers within the same autonomous system (AS).

Key Characteristics

§Uses DUAL (Diffusing Update Algorithm)

finds best path

find backup path

prevent routing loops

§Provides fast convergence

§Uses Bandwidth and Delay to calculate best path

§Maintains Neighbor Table, Topology Table, and Routing Table

Enable EIGRP AS 100 on Routers

Verify EIGRP

Test Connectivity

Task #2: GRE Tunnel Over OSPF (R2 ↔ R3)

Tunnel Network: 10.10.10.0/30

§GRE (Generic Routing Encapsulation) creates a virtual tunnel between R2 and R3

§Runs over the existing OSPF

§networkEncapsulates packets to allow communication between remote networks

Verification

Enable NETCONF on Routers

PYTHON Library used : ncclient

NETCONF Connection Script:

- Connect to router using ncclient manager

- Verify server capabilities

- Close session after testing

TEST NETCONF CONNECTION using Python Script:

from ncclient import manager

router = manager.connect(

host="172.20.0.210",

port=830,

username="wipro",

password="cisco123",

hostkey_verify=False,

allow_agent = False,

look_for_keys = False

)

print("Connected successfully!")

print(router.server_capabilities)

router.close_session()

Python Script  — Push OSPF Using NETCONF

- Connect to router using ncclient manager

- Verify server capabilities

- Use edit-config operation to apply OSPFv2 configuration to the running datastore

- Close session after testing

from ncclient import manager

ROUTER_IP = "172.20.0.210"

USERNAME = "wipro"

PASSWORD = "cisco123"

ospfv2_config = """

<config>

<native xmlns="http://cisco.com/ns/yang/Cisco-IOS-XE-native">

<router>

<ospf xmlns="http://cisco.com/ns/yang/Cisco-IOS-XE-ospf">

<id>1</id>

<router-id>1.1.1.1</router-id>

<network>

<ip>192.168.1.0</ip>

<mask>0.0.0.255</mask>

<area>0</area>

</network>

<network>

<ip>192.168.2.0</ip>

<mask>0.0.0.255</mask>

<area>0</area>

</network>

</ospf>

</router>

</native>

</config>

"""

with manager.connect(

host=ROUTER_IP,

port=830,

username=USERNAME,

password=PASSWORD,

hostkey_verify=False

) as m:

print("✅ Connected to Router3 NETCONF")

response = m.edit_config(target="running", config=ospfv2_config)

print("✅ OSPFv2 Configuration Pushed Successfully!")

print(response)

output :

Python Script 2 — Push OSPFv3 (IPv6) via NETCONF (Router3)

Connects to Router using NETCONF

Sends XML configuration

Enables IPv6 routing

Creates OSPFv3 process 10

Prints confirmation

from ncclient import manager

ROUTER_IP = "172.20.0.210"

USERNAME = "wipro"

PASSWORD = "cisco123"

ospfv3_config = """

<config>

<native xmlns="http://cisco.com/ns/yang/Cisco-IOS-XE-native">

<ipv6>

<unicast-routing/>

<router>

<ospf>

<id>10</id>

</ospf>

</router>

</ipv6>

</native>

</config>

"""

with manager.connect(

host=ROUTER_IP,

port=830,

username=USERNAME,

password=PASSWORD,

hostkey_verify=False

) as m:

print("✅ Connected to Router3 NETCONF")

response = m.edit_config(target="running", config=ospfv3_config)

print("✅ OSPFv3 Process Created Successfully!")

print(response)

OUTPUT:

Manual Configuration vs Automated Configuration

| Aspect | Manual Configuration (CLI Based) | Automated Configuration (NETCONF + Python) |
| --- | --- | --- |
| Configuration Method | Router CLI commands entered manually | Python script using NETCONF (ncclient) |
| Protocols Configured | OSPFv2, OSPFv3, EIGRP, GRE configured one-by-one on each router | OSPFv2 and OSPFv3 pushed automatically via XML |
| Time Required | High – configure each router separately | Low – script pushes config instantly |
| Human Errors | High chance (wrong area, wrong mask, typo errors) | Minimal errors (standardized XML template) |
| Scalability | Difficult for large networks | Easily scalable to many routers |